Copy of the Facility Emergency Response Plan (FRP)

Weld County Pipeline EAP Requirements

PHMSA Regulated Pipeline

(Required documents and files for EAP approval)

Non-PHMSA Regulated Pipeline

(Required documents and files for EAP approval)

- Contact information page

- Project description

- Project Map (Pipeline Route)

- Contact information page

- Project description

- Project Map (Pipeline Route)

Weld County EAP template

Project as-build Shape File (Pipeline Route and aboveground valves)

Project as-build Shape File (Pipeline Route)

Per Weld County Code Division 1 – Regulations for Pipelines - Petroleum Products in the Unincorporated Area of Weld County), Section 21—6-50 a, an approved Emergency Action Plan (EAP) is a permitting requirement for new pipeline projects in the County.

- Pipeline Operators constructing new pipelines that will fall under DOT PHMSA Part 192, Part 194 & 195 requirements shall submit a copy of the Federal Plan to satisfy Weld County’s Pipeline EAP permitting requirement.

- Pipeline Operators constructing new pipelines that do not fall under DOT PHMSA Part 192, Part 194 & 195 requirements shall submit an EAP as part of the Weld County permitting process. The required information can be found in the Pipeline EAP template.

  - Operators may use their own Emergency Planning document; however, the Plan’s Table of Contents shall follow the Pipeline EAP template. This aids first responders, 911 dispatchers, city and county OEMs, and Federal agencies during emergencies to have a consistent Plan structure to follow for emergency response interoperability.

The intent of the pipeline EAP is for emergency planning between Local agencies and Oil and Gas Operators. If applicable, all fire code requirements shall be coordinated with the fire district having jurisdiction. As the final approval signatory, Weld OEM will not delay signing an EAP due to fire code related issues or concerns. Weld County encourages Oil and Gas Operators to work with the local fire districts to ensure they are meeting their adopted fire codes and if applicable, their local jurisdictions permitting requirements.

EMERGENCY ACTION PLAN (EAP)

ABC Energy Company

LOGO

District Office

ABC Energy Company

City, CO, 12345

Proposed Project Start Date:

Table of Contents

SECTION 1: INFORMATION SUMMARY	1

Company Name:	1

24-Hour Emergency Number:	1

Qualified Individual	1

Project Name:	1

Estimated Construction Time:	1

Pipeline Size and Type Description:	1

Product Transported Through Pipeline:	1

Project location narrative:	1

Surrounding Occupancies:	2

Valve Locations:	2

Small Product Release or Spill: 	3

Large Product Release or Spill:	3

SECTION 2: NOTIFICATION PROCEDURES	4

a)	ABC Company (Update and add applicable contact information)	4

b)	Energy Company Community/Media Relations	4

c)	First Responders (Fire, EMS, HazMat)	4

d)	Local, State, and Federal Contacts	4

f)	Spill Response Organization (Contracted)	5

Emergency Notification Priority List:	5

Emergency Notification Checklist:	5

SECTION 3: SPILL DETECTION AND ON-SCENE SPILL MITIGATION PROCEDURES	6

a)	Spill/Release Detection:	6

b)	Spill/Release Response Personnel:	6

c)	Spill/Release Response Equipment:	6

SECTION 4: RESPONSE ACTIVITIES	6

a)	Responsibilities and Actions to be taken by Operating Personnel	6

b)	Qualified Individual's Responsibilities and Authority	6

c)	Coordinating with first responders	6

d)	Spill response organizations	6

SECTION 8 – EVACUATION INFORMATION	8

SECTION 5. TRAINING PROCEDURES	9

SECTION – 10 COORDINATION WITH FIRST RESPONDERS	9

SECTION 6: RESPONSE PLAN REVIEW AND UPDATE PROCEDURES	10

a)	Multi-year plan review and update	10

b)	Post incident plan review and update	10

SECTION 7: MAPS AND DRAWINGS	11

# REVIEW SIGNATURES

| ABC Company |  |  |  |
| --- | --- | --- | --- |
| Name | Signature | Title | Date |
| Fire District |  |  |  |
| Name | Signature | Title | Date |

APPROVAL SIGNATURE

Weld County Office of Emergency Management

Digitally Signed

# SECTION 1: INFORMATION SUMMARY

## Company Name:

ABC Company

Address

City, CO, 1234

Phone number

## 24-Hour Emergency Number:

XXX-XXX-XXXX

## Qualified Individual

Name: John Doe

24-emergency number: xxx-xxx-xxxx

## Project Name:

EXAMPLE – Red/White and Blue pipeline

## Estimated Construction Time:

EXAMPLE – The Red/White and Blue pipeline project is scheduled to break ground in quarter one in 2022 and take 60 days to complete.

## Pipeline Size and Type Description:

EXAMPLE – 24-inch steel pipe with welded joints.

## Product Transported Through Pipeline:

EXAMPLE – Natural Gas will be flowing through the Red/White and Blue pipeline.

## Project location narrative:

EXAMPLE – The Red/White and Blue pipeline will tie into a direct source in the NE quarter of the NW quarter of Section 22, Township 2 North, Range 64 West. From there, it will travel 26 miles to the southwest and tie into a compressor station within the Red/White and Blue NE quarter of the NW quarter of Section 22, Township 2 North, Range 64 West.

The pipeline will travel through four (4) Weld County Fire Districts. Pawnee Fire, Briggsdale Fire, Galeton Fire, and Platte Valley Fire. Please see the overview project map within Annex A of this plan.

## Surrounding Occupancies:

Identify any structures within 150-feet of the proposed pipeline project. Provide the following information for each structure.

- Structure number

- Type of Structure (i.e., Residence, outbuilding, business, etc.)

- Distance from Pipeline

- Address

- Latitude

- Longitude

- Map number which property is identified on

Example

| Structure Number | Type of Structure | Distance from Pipeline (feet) | Address | Latitude | Longitude | Map Number |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Residential | 149 | 123 Cr 19 | 40.000000000 | -104.000000000 | 1 |
| 2 | Outbuilding | 110 | 123 Cr 19 | 40.000000000 | -104.000000000 | 1 |
| 3 | Residential | 149 | 123 Cr 19 | 40.000000000 | -104.000000000 | 1 |
| 4 | Outbuilding | 110 | 123 Cr 19 | 40.000000000 | -104.000000000 | 1 |
| 5 | Outbuilding | 149 | 123 Cr 19 | 40.000000000 | -104.000000000 | 1 |

## Valve Locations:

EXAMPLE

| Valve Type |  |
| --- | --- |
| Valve Location |  |
| GPS | Lat: 40.000000000 Lon: -104.000000000 |
| Valve Type |  |
| Valve Location |  |
| GPS | Lat: 40.000000000 Lon: -104.000000000 |
| Valve Type |  |
| Valve Location |  |
| GPS | Lat: 40.000000000 Lon: -104.000000000 |

Small Product Release or Spill:

Note:  It is generally considered that a Small Release is defined as a release or spill of less than 5 barrels of a “Highly Volatile Liquid.” (For more details see DOT ERG and 49 CFR 195.5).

This can be dependent upon the type of material and should be confirmed through the Environmental Department as to the characteristics of the material as well as the “Reportable Quantity” for the specific released material.

(describe or provide a list of action you will take to respond and mitigate a small product release or spill).

Example – In the event of a small product release or spill, the following steps will be taken to initiate a response and clean up the spill/release:

- Immediately Notify Supervisor, Safety or Control Room.

- If needed, notify 911 and get resource responding.

- Activate the Incident Command System -- Follow directions of Incident Commander.

- Etc.

- Etc.

## Large Product Release or Spill:

(Describe or provide a list of action you will take to respond and mitigate a small product release or spill).

Note:  It is generally considered that a Large Release is defined as a release or spill of 5 barrels or more, of a “Highly Volatile Liquid;” or any product or chemical with characteristics that create immediate safety/health hazards or will migrate off-site.

(For more details see DOT ERG and 49 CFR 195.5)

This can be dependent upon the type of material and should be confirmed through the Environmental Department as to the characteristics of the material as well as the “Reportable Quantity” for the specific released material.

Example – In the event of a large product release or spill, the following steps will be taken to initiate a response and clean up the spill/release:

- Immediately Notify Supervisor, Safety or Control Room.

- If needed, notify 911 and get resource responding.

- Activate the Incident Command System -- Follow directions of Incident Commander.

- Etc.

- Etc.

# SECTION 2: NOTIFICATION PROCEDURES

Example narrative and contact list: In the event a release from the (pipeline name) the following procedures listed below will be followed to effectively report the incident and to mobilize the necessary equipment and personnel to respond and mitigate the incident. (below is an example list)

## ABC Company (Update and add applicable contact information)

| Name | Office Phone | Emergency/Cell |
| --- | --- | --- |
| 24-Emegency number | N/A | xxx-xxx-xxxx |
| Qualified Individual | xxx-xxx-xxxx | xxx-xxx-xxxx |
| Energy Company EHS on-call Emergency Number | xxx-xxx-xxxx | xxx-xxx-xxxx |
| EHS Supervisor | xxx-xxx-xxxx | xxx-xxx-xxxx |
| EHS – Safety | xxx-xxx-xxxx | xxx-xxx-xxxx |
| EHS – Environmental | xxx-xxx-xxxx | xxx-xxx-xxxx |

## Energy Company Community/Media Relations

| Name | Office Phone | Cell Phone |
| --- | --- | --- |
| Personnel Name | xxx-xxx-xxxx | xxx-xxx-xxxx |
| Personnel Name | xxx-xxx-xxxx | xxx-xxx-xxxx |

## First Responders (Fire, EMS, HazMat)

| Name | Emergency Number | Non-Emergency Number |
| --- | --- | --- |
| *All emergency notifications require notification to 911 first |  |  |
| Platteville – Gilcrest Fire Protection District – Station #1 | 911 |  |
| Platteville – Gilcrest Fire Protection District – Station #1 | 911 |  |
| Weld County Sheriff | 911 | 970-356-4015 |
| Colorado State Highway Patrol | 911 | 970-506-4999 |
| Platteville Police Department | 911 | 970-785-2215 |

## Local, State, and Federal Contacts

| Name | Emergency Number | Non-Emergency Number |
| --- | --- | --- |
| Weld County Oil and Gas Energy Department | none | 970-400-3580 |
| Weld County Office of Emergency Management | 911 | 970-400-3992 |
| CDPHE | none | 877-518-5608 |
| Colorado Parks & Wildlife | none | 303-291-7227 |
| National Response Center | 800-424-8802 | none |

- Hospitals (Minimum 2 closest locations to site)

| Name | Office Phone |
| --- | --- |
| Northern Colorado Medical Center | 970-352-4121 |
| Medical Center of the Rockies | 970-624-2500 |

## Spill Response Organization (Contracted)

| Name | 24/7 Emergency Number | Non-Emergency Number |
| --- | --- | --- |
| Third Party Contractor | xxx-xxx-xxxx | xxx-xxx-xxxx |
| Third Party Contractor | xxx-xxx-xxxx | xxx-xxx-xxxx |

## Emergency Notification Priority List:

(Provide a priority list of notifications).

Example priority Notification list

| Priority | Notification |
| --- | --- |
| 1 | 911 |
| 2 | Qualified Person |
| 3 | Spill Response Team |
| 4 | Spill Response Clean-up Company |
| 5 | National Response Center |
| 6 | Etc. |
| 7 | Etc. |

## Emergency Notification Checklist:

(Provide a sample notification priority checklist).

Example priority Notification checklist

| Priority | Notification | Time | Check |
| --- | --- | --- | --- |
| 1 | 911 |  |  |
| 2 | Qualified Person |  |  |
| 3 | Spill Response Team |  |  |
| 4 | Spill Response Clean-up Company |  |  |
| 5 | National Response Center |  |  |
| 6 | Etc. |  |  |
| 7 | Etc. |  |  |

Information to be provided in the initial and each follow-up notification: (describe or provide a list of actions you will take to respond and mitigate a small product release or spill)

Example of initial and follow-up information

| Name of pipeline |  |
| --- | --- |
| Time of discharge |  |
| Location of discharge |  |
| Name of product involved |  |
| Reason for discharge (e.g., material failure, excavation damage, corrosion) |  |
| Estimated volume of product discharged |  |
| Weather conditions on scene |  |
| Actions taken or planned by persons on scene |  |

# SECTION 3: SPILL DETECTION AND ON-SCENE SPILL MITIGATION PROCEDURES

## Spill/Release Detection:

Describe in detail how you will discover a spill or release of the product within the proposed pipeline.

## Spill/Release Response Personnel:

Describe in detail the procedures and the order of priority in which company ABC personnel and contactors are required to follow while responding to a pipeline emergency to mitigate or prevent any discharge from the pipeline.

## Spill/Release Response Equipment:

- Provide a list of equipment that may be needed in response activities on land and navigable waters, including – transfer hoses and connection equipment, portable pumps and ancillary equipment, and facilities available to transport and receive oil from a leaking pipeline.

- Identification of the availability, location, and contact telephone numbers to obtain equipment for response activities on a 24-hour basis.

- Identification of personnel and their location, telephone numbers, and responsibilities for use of equipment in response activities on a 24-hour basis.

# SECTION 4: RESPONSE ACTIVITIES

## Responsibilities and Actions to be taken by Operating Personnel

Describe the responsibilities and actions to be taken by operating personnel to initiate and supervise response actions pending the arrival of the qualified individual or other response resources identified in the response plan.

## Qualified Individual's Responsibilities and Authority

Describe the qualified individual's responsibilities and authority, including notification of the response resources identified in the plan.

## Coordinating with first responders

Describe the procedures for coordinating the actions of the operator or qualified individual within a Unified Command structure with first responders.

## Spill response organizations

Describe the spill response organizations available, through contract or other approved means, to respond to a spill/release to the maximum extent practicable.

- For each organization identified under paragraph (d) of this section, a listing of:

- Equipment and supplies available

- Trained personnel necessary to continue operation of the equipment and staff the oil spill removal organization for the first 7 days of the response.

- Spill Reporting

(Edit as needed, EXAMPLE) The person reporting a spill may be required to supply the minimum spill assessment information to provide as a complete understanding of the incident as possible to local, state, or federal agencies if applicable. Some initial spill response actions and information that may be reported are presented below:

- A spill/release will be reported to the ECMC if released material is property of Energy Company and meets the ECMC reporting thresholds (see below), an example would be crude oil released from a separator or produced water from a water vault.

- A spill/release will be reported to the Weld County LEPC if released material is property of Energy Company and meets the ECMC reporting thresholds (see below) Mandated by Section 304 of the Emergency Planning and Community Right-To-Know Act (EPCRA).

A spill/release will be reported to the CDPHE if released material is in the custody of a third party for spills meeting CDPHE reporting thresholds or are of any size that impact or threaten to impact waters of the state, a residence or occupied structure, livestock or public byway. An example would be an oil hauler over filling a truck and spills product onto the ground next to a flowing irrigation ditch.

Spills are reportable to the ECMC and/or to Weld County LEPC in the following circumstances:

- the spill or release impacts or threatens to impact any waters of the state, a residence or occupied structure, livestock or a public byway;

- a spill or release in which 1 barrel or more is released outside of berms or other secondary containment.

- or any spill or release of 5 barrels or more. If the spill impacts or threatens to impact waters of the state (which include surface water, ground water and dry gullies or storm sewers leading to surface water), it must also be reported immediately to CDPHE (25-8-601 CRS). Petroleum releases of 25 gallons or more.

Once a spill is determined reportable, there is a 24-hour deadline to make initial notification to the ECMC or CDPHE and the Weld County LEPC depending on product ownership. Spills/releases in the custody of Energy Company will be reported by a Company representative. Spills/releases in the custody of a third party will be reported by the responsible company’s EHS Department to the appropriate agency and to Energy Company.

These regulatory guidelines will be strictly followed by Energy Company and any contractors operating under Energy Company guidance during all activities at the Pad Name and Legal Location.

# SECTION 8 – EVACUATION INFORMATION

Evacuation Plan Procedures (public)

(Edit as needed, EXAMPLE) The procedure to be used in alerting the public in the event of than incident which could pose a threat to life or property will be arranged and coordinated with first responders and Weld County Emergency Management.

In the event of an actual emergency, the following steps will be immediately taken:

- The Energy Company representative will immediately notify first responders (911), to warn the public of a potential chemical exposure.

- First responders may conduct door to door evacuation notices in addition to reverse 911 and utilizing the Integrated Public Alert and Warning System (IPAWS).

- The Energy Company responsible for employees and contract personnel will monitor essential and non-essential personnel traffic on or near the incident site.

- General:

  - The area included within the radius of exposure is considered to be the zone with the maximum potential hazard, per the Emergency Response Guide (ERG). When it is determined that conditions exist which create an additional area (beyond the initial zone of maximum potential hazard) vulnerable to possible hazard, public areas in the additional hazardous area will be evacuated.

  - In the event of an incident, after the public areas have been evacuated and traffic stopped, it is expected that local civil authorities will have arrived and within a few hours will have assumed direction of and control of the public, including all public areas.

  - Energy Company will cooperate with these authorities to the fullest extent and will exert every effort by careful advice to such authorities to prevent panic or rumors.

Energy Company will dispatch appropriate personnel to the disaster site as soon as possible. The company’s personnel will cooperate with and provide such information to civil authorities as they might require.

# SECTION 5. TRAINING PROCEDURES

Include a description of the training procedures and programs of the operator.

The National Incident Management System (NIMS) guides all levels of government, non-governmental organizations and the private sector to work together to prevent, protect against, mitigate, respond to and recover from incidents.

NIMS provides stakeholders across the whole community with the shared vocabulary, systems and processes to successfully deliver the capabilities described in the National Preparedness System. NIMS defines operational systems that guide how personnel work together during incidents.

Energy Company plays a vital role in the Incident Management System. Energy Company has a significant impact on local, regional, and national economic recovery, and is part of the whole community and essential to the function of the Community Lifelines.

To maximize Energy Company’s impact and willingness to participate in incident operations, Energy Company should coordinate and integrate with first responders into a Unified Command —including planning, training, and preparedness exercises. In addition, it is also recommended that all Energy Company employees who will respond to an incident have training in ICS 100, ICS 200, and ICS 700 at a minimum.

EXERCISES: (Edit as needed, EXAMPLE) Exercises are an important component to test an organization's response readiness, training and familiarity with various emergency response scenarios, participation and engagement with local and or state agencies, and to develop lessons learned to improve emergency response capabilities. Per COGCC guidance number 16, a proposed schedule and type of exercises are provided below:

# SECTION – 10 COORDINATION WITH FIRST RESPONDERS

- (Edit as needed, EXAMPLE) Energy Company will communicate site construction, drill spud, completion operations, and Production Turn-In-Line dates to the Weld County Office of Emergency Management for coordination and communication with local first responders. These starting dates will be provided with a minimum of 7 business days prior to commencement or any change in oil and gas development operations.

- (Edit as needed, EXAMPLE) Energy Company EHS representative and first responders identified in this Emergency Action Plan and Tactical Response Plan have reviewed both documents and have discussed coordination efforts in the event of an emergency requiring first responder assistance.

- (Edit as needed, EXAMPLE) In the event of an emergency requiring First Responders, Unified Command will be established between the Energy Company appointed company man on location and First Responders having jurisdiction, including CDPHE and ECMC.

  - Unified Command Post:

Energy Company will establish a single Unified Command Post with first responders that will be established based on conditions present at the time of the incident. The Unified Command Post will be staffed with Command Staff and General Staff to reduce duplication and streamline decision-making.

  - Incident Management Team Activation:

Depending on the complexity of the incident, Energy Company and first responders will define clear triggers for Incident Management Team activation.

  - ICS Documentation Standards:

Energy Company will transition from ICS 201 to a full Incident Action Plan (IAP) when an incident extends beyond the initial operational period.

Energy Company will use standard ICS forms during incident stabilization before transitioning to corporate systems for recovery.

# SECTION 6: RESPONSE PLAN REVIEW AND UPDATE PROCEDURES

## Multi-year plan review and update

Describe the procedures to update this plan every five (5) years or after a change in personnel, operations or ownership.

## Post incident plan review and update

Describe the procedures to update this plan after a spill/release in order to evaluate and record the plan's effectiveness.

# SECTION 7: MAPS AND DRAWINGS

Provide maps illustrating the pipeline project plan of travel.